DHCP Server Conf:

ip dhcp excluded-address low [high]

ip dhcp pool POOL-NAME

network network-number [mask | /prefix-length]

default-router address [address2...address8]

dns-server address [address2...address8]

lease {days [hours] [minutes] | infinite}

no service dhcp	//disable

service dhcp		//re-enable

Verify:

show ip dhcp binding

show ip dhcp server statistics

show ip dhcp pool

For Host

ipconfig /all

ipconfig /renew

ipconfig /release

DHCP Relay:

ip helper-address ip addr 	//interface mode //in that int where there is no dhcp server

DHCP Client as a Router

int f0/0

ip address dhcp

no shut

NAT

Static:

Router(config)#ip nat inside source static local-ip global-ip

Router(config)#interface type number

Router(config-if)#ip nat inside

Router(config)#interface type number

Router(config-if)#ip nat outside

Dynamic

Router(config)#ip nat pool name start-ip end-ip {netmask netmask|prefix-length prefix-length}

Router(config)#access-list access-list-number permit source [source-wildcard]

Router(config)#ip nat inside source list access-list-number pool name

Router(config)#interface type number

Router(config-if)#ip nat inside

Router(config)#interface type number

Router(config-if)#ip nat outside

Overload:

ip nat inside source list acl-number interface interface  overload	//in case of one ip address

ip nat inside source list acl-number  pool name overload 	//for a pool

Verify:

show ip nat translations [erbose]

show ip nat statistics

clear ip nat translation * 	//all clear